| Interplanetary phase | TCM1 = 0.00780 [km/s] TCM2 = 0.000751 [km/s] |
| --- | --- |
| M.I.O | Extra man = 0.0010 [m/s] MIOM = 1.005 [km/s] |
| AEROBRAKING | ABM = 0.019 [km/s] ABX = 0.025 [km/s] |
| TRANSITION | OA-1 = 0.0150 [km/s] OA-2 = 0.0535 [km/s] |

In this section, an analysis and evaluation of the mission trajectory design is performed, aimed at

calculating and justifying the costs’ results in terms of ∆V , using a reverse engineering approach to the

main orbital maneuvers and overall station maintenance. Using the method of patched conics and an

impulsive maneuver approach, the main maneuvers reported in the previous section can be evaluated

with their relative ∆V . For the MOI maneuver, the velocity at enter of the sphere of influence is

evaluated by algorithm based on Lambert problem, then the ∆V to put the spacecraft in the insertion

orbit is computed as an impulsive maneuver execute at the pericenter of the entry hyperbola, located

in corrispondence of the pericenter of the inserton orbit, resulting equal to ∆V =952.9 m/s. This result

is lower than the real one (∆V = 1000.5 m/s) because in the real case is not possible an impulsive

maneuver and the distribution in time of the thrust will require an higher ∆V to compensate effects

due to gravitational attraction. For the maneuvers two maneuvers ABMs, a similar approach is

improve, considering impulsive maneuver executed at the apoapsis to lower the periapsis, with the

following results ∆V (ABM-1)=4.1 m/s and ∆V (ABM-2)=4.2 m/s, where the remaining four ABMs,

that lower the periapsis up to 108 km introduce a ∆V = 6 m/s. Regarding the aerobraking phase, an

estimation of the saved ∆V can be performed by an energy balance between the initial orbit, with

apoapsis altitude of 44982 km, and the final one, with apoapsis at 486 km from the surface, keeping

fix the pericenter and introducing the effect of aerobraking as an impulsive maneuver. The resulting

∆V equal to 1994 deviate from the real one, ∆V =1200 m/s, probabily due to ABM maneuvers

executed during the aerobraking, from which the final orbit result with a periapsis altitude of 105.3

km. The remaing maneuvers executed along the trajectory, evaluated by the same previous approach,

results as follow: ∆V (ABX) = 26.2 m/s, ∆V (AO-1)= 9.3 m/s, ∆V (AO-2)= 37.1 m/s, that result

different from the real one according with the previous consideration and a contribution related to

Mars atmospheric perturbation.